Devin Fromond

Lab Report

ECE 2031 L04

22 October 2025

Figure 1. SCOMP processor simulator showing the custom assembly program assembled and executing within the infinite loop, confirming correct operation of the LOADI, SUB, and STORE instructions.

; SimpleSubAndStore.asm (ASM)

; Load 20, sub contents of 0x1F, store to 0x20, loop forever

; Devin Fromond

; ECE2031 L4
; 10/18/2025

ORG     0

Start:      LOADI   20          ; AC = 20

SUB     Subtrahend  ; AC = 20 - [0x001F]

STORE   Result      ; [0x0020] = AC

Loop:       JUMP    Loop        ; Infinite loop

ORG     &H001F

Subtrahend: DW      30     ; init [0x001F] with a value > 25

ORG     &H0020

Result:     DW      0           ; Storage at [0x0020]

Figure 2. ASM code used to demonstrate functionality of load, subtract, and store operations.

Figure 3. Waveform simulation demonstrating working JPOS (Jump POSitive) and JNZ (Jump Not Zero) functionality (best observed on the dbg_AC line) for the corresponding VDHL file as in Appendix A.

Figure 4. Waveform simulation demonstrating working SUB (SUBtract) functionality (best observed on the dbg_AC) for the corresponding VDVL file as in Appendix A.

APPENDIX A

VDHL FILE CONTAINING ADDED JPOS/JNZ/SUB OPCODES AND FUNCTIONALITY

-- SCOMP.vdh

-- Basic Opcode defs and jpos/jnz/sub functionality

-- Devin Fromond

-- ECE 2031 L04

-- 10/18/2025

-- SCOMP, the Simple Computer.

-- A 16-bit computer designed to be easy to design and modify.

-- Updated 2025

library altera_mf;

library lpm;

library ieee;

use altera_mf.altera_mf_components.all;

use lpm.lpm_components.all;

use ieee.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

use ieee.std_logic_arith.all;

entity SCOMP is

port(

clock     : in      std_logic;

resetn    : in      std_logic;

IO_READ   : out     std_logic;

IO_WRITE  : out     std_logic;

IO_ADDR   : out     std_logic_vector(10 downto 0);

IO_DATA   : inout   std_logic_vector(15 downto 0);

dbg_FETCH : out     std_logic;

dbg_AC    : out     std_logic_vector(15 downto 0);

dbg_PC    : out     std_logic_vector(10 downto 0);

dbg_NMA   : out     std_logic_vector(10 downto 0);

dbg_MD    : out     std_logic_vector(15 downto 0);

dbg_IR    : out     std_logic_vector(15 downto 0)

);

end SCOMP;

architecture a of SCOMP is

type state_type is (

init, fetch, decode,

ex_nop, ex_load, ex_store, ex_store2,

ex_loadi, ex_add, ex_sub, ex_jpos, ex_jnz, ex_addi,

ex_and, ex_or, ex_xor, ex_shift,

ex_jump, ex_jneg, ex_jzero,

ex_return, ex_call,

ex_in, ex_in2, ex_out, ex_out2

);

-- custom type for the call stack

constant STACK_SIZE  :  integer := 10;

type stack_type is array (0 to STACK_SIZE) 
     of std_logic_vector(10 downto 0);

-- internal signals

signal state         :  state_type;

signal AC            :  std_logic_vector(15 downto 0);

signal AC_shifted    :  std_logic_vector(15 downto 0);

signal PC_stack      :  stack_type;

signal IR            :  std_logic_vector(15 downto 0);

signal mem_data      :  std_logic_vector(15 downto 0);

signal PC            :  std_logic_vector(10 downto 0);

signal next_mem_addr :  std_logic_vector(10 downto 0);

signal operand       :  std_logic_vector(10 downto 0);

signal MW            :  std_logic;

signal io_drive_en   :  std_logic;

begin

-- use altsyncram for unified program and data memory

altsyncram_component : altsyncram

GENERIC MAP (

numwords_a => 2048,

widthad_a => 11,

width_a => 16,

init_file => "SimpleDemo.mif",

clock_enable_output_a => "BYPASS",

lpm_hint => "ENABLE_RUNTIME_MOD=NO",

intended_device_family => "CYCLONE V",

clock_enable_input_a => "BYPASS",

lpm_type => "altsyncram",

operation_mode => "SINGLE_PORT",

power_up_uninitialized => "FALSE",

read_during_write_mode_port_a=>"NEW_DATA_NO_NBE_READ",

outdata_reg_a => "UNREGISTERED",

outdata_aclr_a => "NONE",

width_byteena_a => 1

)

PORT MAP (

wren_a    => MW,

clock0    => clock,

address_a => next_mem_addr,

data_a    => AC,

q_a       => mem_data

);

-- use Intel IP to shift AC for shift instruction

shifter: lpm_clshift

generic map (

lpm_width     => 16,

lpm_widthdist => 4,

lpm_shifttype => "arithmetic"

)

port map (

data      => AC,

distance  => IR(3 downto 0),

direction => IR(4),

result    => AC_shifted

);

-- Memory address comes from PC during fetch

with state select next_mem_addr <=

PC when fetch,

operand when others;

-- This makes the operand available immediately after fetch

with state select operand <=

mem_data(10 downto 0) when decode,

IR(10 downto 0) when others;

-- use lpm tri-state driver to drive i/o bus

io_bus: lpm_bustri

generic map (

lpm_width => 16

)

port map (

data     => AC,

enabledt => io_drive_en,

tridata  => IO_DATA

);

-- IR has the IO address during INs and OUTs

IO_ADDR  <= IR(10 downto 0);

process (clock, resetn)

begin

if (resetn = '0') then          -- Active-low reset

state <= init;

elsif (rising_edge(clock)) then

case state is

when init =>

MW        <= '0';   -- clear mem w flag

PC        <= "00000000000"; -- reset PC

AC        <= x"0000";       -- clear AC

io_drive_en <= '0';  -- don't drive IO

state     <= fetch;   -- start cycles

when fetch =>

io_drive_en <= '0'; -- stop driving IO

PC        <= PC + 1;   -- increment PC

state     <= decode;

when decode =>

IR    <= mem_data; -- latch inst

case mem_data(15 downto 11) is

when "00000"  =>  -- nop

state <= ex_nop;

when "00001"  =>        -- load

state <= ex_load;

when "00010"  =>        -- store

MW    <= '1';  -- mem wr

state <= ex_store;

when "00011"  =>        -- loadi

state <= ex_loadi;

when "00100"  =>        -- add

state <= ex_add;

when "00101"  =>        -- sub

state <= ex_sub;

when "00110"  =>        -- addi

state <= ex_addi;

when "00111"  =>        -- and

state <= ex_and;

when "01000"  =>        -- or

state <= ex_or;

when "01001"  =>        -- xor

state <= ex_xor;

when "01010"  =>        -- shift

state <= ex_shift;

when "01011"  =>        -- jump

state <= ex_jump;

when "01100"  =>        -- jneg

state <= ex_jneg;

when "01101"  =>        -- jpos

state <= ex_jpos;

when "01110"  =>        -- jzero

state <= ex_jzero;

when "01111"  =>        -- jnz

state <= ex_jnz;

when "10000"  =>        -- call

state <= ex_call;

when "10001"  =>      -- return

state <= ex_return;

when "10010"  =>        -- in

state <= ex_in;

when "10011"  =>        -- out

state <= ex_out;

io_drive_en <= '1';

when others =>

state <= ex_nop;

end case;

when ex_nop =>

state <= fetch;

when ex_load =>

AC    <= mem_data; -- latch data

state <= fetch;

when ex_store =>

MW    <= '0'; -- drop MW to end write

state <= fetch;

when ex_loadi =>             -- sign-extend

AC    <= (IR(10) & IR(10) & IR(10) &

IR(10) & IR(10) & IR(10 downto 0));

state <= fetch;

when ex_add =>

AC    <= AC + mem_data;   -- addition

state <= fetch;

when ex_addi =>

-- sign extension

AC    <= AC + (IR(10) & IR(10) &IR(10 &

IR(10) & IR(10) & IR(10 downto 0));

state <= fetch;

when ex_jump =>

PC    <= operand  -- overwrite PC

state <= fetch;

when ex_jneg =>

if (AC(15) = '1') then

PC    <= operand;

end if;

state <= fetch;

when ex_jzero =>

if (AC = x"0000") then

PC    <= operand;

end if;

state <= fetch;

when ex_and =>

AC    <= AC and mem_data; --  AND

state <= fetch;

when ex_or =>

AC    <= AC or mem_data;

state <= fetch;

when ex_xor =>

AC    <= AC xor mem_data;

state <= fetch;

when ex_shift =>

AC    <= AC_shifted;

state <= fetch;

when ex_call =>

for i in 0 to STACK_SIZE-1 loop

PC_stack(i + 1) <= PC_stack(i);

end loop;

PC_stack(0) <= PC;

PC          <= operand;

state       <= fetch;

when ex_return =>

for i in 0 to STACK_SIZE-1 loop

PC_stack(i) <= PC_stack(i + 1);

end loop;

PC          <= PC_stack(0);

state       <= fetch;

when ex_in =>

IO_READ <= '1'

state <= ex_in2;

when ex_in2 =>

IO_READ <= '0';

AC <= IO_DATA;

state <= fetch;

when ex_out =>

IO_WRITE <= '1';

state <= ex_out2;

when ex_out2 =>

IO_WRITE <= '0';

state <= fetch;

-- SUB: AC <= AC - MEM(Address)

when ex_sub =>

AC    <= AC - mem_data;

state <= fetch;

-- JPOS: If AC > 0 , jump.

when ex_jpos =>

if (AC(15)='0' and AC/= x"0000") then

PC <= operand;

end if;

state <= fetch;

-- JNZ: If AC /= 0, jump.

when ex_jnz =>

if (AC /= x"0000") then

PC <= operand;

end if;

state <= fetch;

when others =>

state <= init;

end case;

end if;

end process;

-- Additional outputs to aid simulation

dbg_FETCH <= '1' when state = fetch else '0';

dbg_PC    <= PC;

dbg_AC    <= AC;

dbg_IR    <= IR;

dbg_NMA   <= next_mem_addr;

dbg_MD    <= mem_data;

end a;

APPENDIX B

ASM FILE FOR AN SCOMP BYTE-DIFFERENCE PROGRAM

; ByteDiff.asm

; Compute |high_byte(Value) - low_byte(Value)| and st in Result

; Devin Fromond

; ECE 2031 L04

; 10/18/2025

ORG 0

Start:

; --- Grab HIGH BYTE into High

LOAD   Value          ; AC = 0xHHLL

AND    HiMask         ; AC = 0xHH00

SHIFT  -8             ; arithmetic >>8 may sign-extend...

AND    LowMask        ; AC = 0x00HH

STORE  High

; --- Grab LOW BYTE into Low

LOAD   Value          ; AC = 0xHHLL

AND    LowMask        ; AC = 0x00LL

STORE  Low

; --- Compute (High - Low)

LOAD   High           ; AC = High

SUB    Low            ; AC = High - Low

JNEG   MakePos        ; if negative, negate it

STORE  Result         ; already positive

JUMP   Finish

MakePos:

STORE  Diff           ; Diff = (High - Low) < 0

LOADI  0

SUB    Diff           ; AC = -Diff = |High - Low|

STORE  Result

Finish:

JUMP   Finish         ; Infinite loop

Value:     DW   &H89F1    ; Start with 0x89F1 (expected 104)

; Test Case: HF000  (expected 240)

; Test Case: HFFFF  (expected 0)

; Test Case: H1248  (expected 54)

Result:    DW   0

High:      DW   0

Low:       DW   0

Diff:      DW   0

LowMask:   DW   &H00FF

HiMask:    DW   &HFF0